LYNMOUTH

HARBOUR

EMERGENCY

RESPONSE

PLAN

Adopted by Cllr David Clayton as leader of the Full Council, PMSC Duty Holder on 23rd September 2026

Contents

i Document Control	6

ii Record of Review /Amendments	6

iii Distribution List	7

iv Definitions of Abbreviations	8

V lYNMOUTH Harbour	9

Consultation	9

vi The Harbour Organisational Chart	10

vii The Harbour Jurisdiction & Map	10

1.0 Introduction	11

1.1 Aim	11

1.2 Objective	11

1.3 Overall Operation	11

1.4 Exercising the Plan.	11

2.0 Action	11

2.1 Types of Potential Incidents	12

2.1.1 Potential Sources of Incidents	13

2.3 Dangerous Goods	13

3.0 Raising the Alarm.	13

3.1 Means of Raising the Alarm	14

4.0 Incidents within the Jurisdiction	14

4.1 Responsibilities’	15

4.2 General Guidance Points	15

5.0 Fires	15

5.1 General	15

5.2 Fire on Vessels	15

5.3 Fire on Land	16

5.4 Action by vessel	16

5.5 Access to site	16

5.5.1 Personnel on site	16

5.5.2 Incident Report & Log	16

5.5.3 Risk assessment.	16

5.5.4 A site survey analysis.	17

5.6 Action by the Fire and Rescue Service	17

5.7 Action by the police	18

5.7.1 Inner Cordon	18

5.7.2 Outer Cordon	18

5.7.3 Operations Analysis	19

5.8 Action by Coastguard	19

6.0 Medical Emergencies	19

6.1 Medical Emergencies on board	19

6.2 Action by Medical Services	20

6.3 Helicopter Operations & Landing Sites	20

6.3.1 Night operations.	20

7.0 Evacuation Procedures	21

8.0 Subsidiary Considerations:	22

9.0 Pollution	23

9.1 Actions by Vessels	23

9.1.1 Reporting of pollution	23

9.1.2 Method of reporting	23

9.2 Actions on receipt of Pollution Report	23

9.2.1 Harbour Authority	23

9.2.2 Shore Operator	24

9.2.3 Vessel	24

9.3 Containment of Pollution	24

9.4 Damaged/Leaking vessel wishing to enter Harbour	24

9.5 Pollution entering lYNMOUTH Harbour from seaward	24

10.0 Recovery & Salvage Operations	24

11.0 Personal Protective Equipment (P.P.E.)	26

12.0 Bodies in the Harbour	26

13.0 Animals	26

14.0 On scene management	26

14.1 Transfer of Command Post	26

14.2 Emergency Response Control Centre (E.R.C.C.)	26

14.3 Environment Agency (E.A.)	29

14.4 Secretary of State’s Representative (S.O.S.R.E.P.)	29

14.5 Emergency Response Centres	30

14.6	Support Services	30

15.0 Key Considerations	30

15.1 Media & Communications	30

15.2 Means of communication.	31

15.2.1 Mobile phone communication.	31

15.3 Contact with Key Service Providers	32

15.3.1 Insurance	32

15.3.2 Harbour Users / Tenants of Harbour Properties	33

15.3.3 Government Authorities	33

Annexes	34

1.0 Contact Directory	34

2.0 Templates	37

2.1 Incident Report Form	37

2.2 Incident Log	38

2.3 Resources used in Incident response Form	39

2.4 Press Release Form	40

2.5 Dynamic Risk Assessment Form	41

2.6 CG77 POLREP	44

2.7 JESIP M/THANE Form	45

3.0 Action Response Flow Charts	46

3.1 Fire On Quayside – General.	46

3.2 Fire – Ship Whilst Alongside Berth	47

3.3 Fire / Grounding / Sinking Or Capsizing Of A Vessel in Harbour Limits.	48

3.4 Actions Assuming Vessel Not Berthing	48

3.5 Actions If Vessel Is Able To Come Alongside A Berth	49

3.6 Fire Or Explosion On A Vessel Carrying Explosive/Hazardous Materials	50

3.7 Incident Involving Damage to Harbour Infrastructure	52

Appendices	53

1.0 Dangerous Goods / Explosive Placards	53

1.1 I.M.D.G. Codes & Interpretation.	53

2.0 Dangerous Goods – Consequences of Incidents	54

2.1 Non-Toxic & Non-Flammable Gases.	54

2.2 Poison Gases.	54

2.3 Flammable Liquids.	55

2.4 Flammable Solids.	55

2.5 Spontaneous Combustible.	56

2.5.1 Substances Liable to Spontaneous Combustion.	56

2.6 Dangerous When Wet.	56

2.7 Oxidizers.	57

2.8 Organic Peroxides.	57

2.9 Poisons.	58

2.10 Infectious Substances.	58

2.11 Radioactive.	58

2.12 Corrosive.	59

2.13 Miscellaneous Dangerous Substances.	59

3.0 References	60

# i Document Control

This Plan will be reviewed periodically and at periods of no greater than 2 years.

It shall also be reviewed following any significant changes in Harbour operations or after relevant exercises or real-life incidents, to ensure that it still covers all potential scenarios within the Harbour area.

Major changes will be issued as a new version with all minor amendments to that version annotated by a decimal point e.g. V4 will become V4.1, V4.2 etc.

Amendment proposals should be sent to the Town Clerk using the following means:

Email: LTC@northdevon.gov.uk

Phone: 01598 752384

Post: Town Clerk, Town Hall, Lee Road, Lynton. EX35 6HT

# ii Record of Review /Amendments

| Review (R) or Amendment (A) | Date | Version # | Page | Description of changes |
| --- | --- | --- | --- | --- |
| New Version | 12/05/2026 | 1.0 | All | New Version |
| R&A | 25/04/2026 | 1.1 | All | Naming realigned and Ilfracombe references removed. |

# iii Distribution List

| Town Clerk | 1 | Hard/Electronic |
| --- | --- | --- |
| Lynton Town Hall (Reception) | 2 | Electronic |
| Lynton & Lynmouth Town Council website – Harbour Page | 3 | Electronic |
| PMSC - Designated Person | 4 | Electronic |
| Ambipar | 5 | Electronic |

# iv Definitions of Abbreviations

| ACRONYM | MEANING |
| --- | --- |
| AED | Automated External Defibrillators |
| AIC | Ambulance Incident Commander |
| BEC | Brynsworthy Environment Centre |
| CPSO | Counter Pollution & Salvage Officer |
| DCC | Devon County Council |
| DEFRA | Department for Environment, Food & Rural Affairs |
| DfT | Department for Transport |
| DGHAR | Dangerous Goods in Harbour Areas Regulations 2016 |
| EOD | Explosive Ordnance Disposal Team |
| ERCC | Emergency Response Command Centre |
| FCP | Forward Command Post |
| FRS | Fire and Rescue Services |
| HM | Marine Advisor |
| ICV | Incident Control Vehicle |
| ITC | Ilfracombe Town Council |
| MCA | Maritime & Coastguard Agency |
| MIMMS | Major Incident Medical Management & Support |
| NDC | North Devon Council |
| OSCP | Oil Spill Contingency Pan |
| R.N.L.I | Royal National Lifeboat Institution |
| SOPEP | Shipboard Oil Pollution Emergency Plan |
| SOSREP | Secretary of State Representative |
| SWAST | South West Ambulance Service Foundation Trust |

O

# v Lynmouth Harbour

Lynmouth is a harbour on the North Devon coast where Exmoor joins the sea. From Lynmouth one can reach other harbours along the North Devon and Bristol Channel coasts.

Lynmouth Harbour is a manmade harbour that provides good shelter from the majority of weather systems experienced in the U.K

Lynmouth does not handle hazardous cargo in so far as shipping is concerned.

The Harbour Owner, North Devon Council is a licensed Category 1 responder under the Civil Contingency Act and as such has Statutory Duties.

https://www.legislation.gov.uk/ukpga/2004/36/schedule/1

## Consultation

In drawing up this emergency plan the following have been consulted:

- North Devon District Council.

- H.M. Coastguard.

- Lynmouth Harbour Users Group

- Lynmouth Sailing Club

- Lynton & Lynmouth Town Council.

Note: In the operation of this plan, it must be noted that the territorial limits of the Harbour are as outlined within the map as provided in section vii. Any oil spill or incident involving a vessel within this area falls to the responsibility of the Harbour owner.

# vi The Harbour Organisational Chart

# vii The Harbour Jurisdiction & Map

(Awaits formalised Harbour Empowerment Order to delineate official boundaries)

# 1.0 Introduction

This plan has been compiled and published in accordance with The Port & Marine Facilities Safety Code 2025.

## 1.1 Aim

The Aim of this plan is to provide an easy source of reference to those dealing with an emergency within the Harbour Area.

## 1.2 Objective

The Objective of the plan is to ensure all agencies respond to an emergency within the Harbour Area in a coordinated manner.

## 1.3 Overall Operation

The DGHAR 2016 require Harbour Authorities to have an effective Emergency Plan in place to deal with emergencies involving dangerous substances within the Harbour or Harbour area.

Although Lynmouth Harbour does not have a designated Harbour Authority at this time, the Harbour owners, North Devon Council have implemented this Emergency Response Plan as a matter of best practice.

This plan extends to include other potential major incident scenarios that would require the implementation of a large-scale emergency response. This is to be construed as any one of the following styles of incidents:

- Fire/explosion on a vessel alongside a quay or whilst moving within the Harbour’s confines.

- Any vessel in distress approaching the Harbour.

- A vessel sinking within the Harbour’s confines.

- A vessel capsizing within the Harbour’s confines.

- A vessel stranding within the Harbour’s confines.

- Vessels colliding within the Harbour’s confines.

- Major oil or chemical leakage / spill into harbour.

- Escape of toxic and / or flammable gas forming vapour cloud.

- Extreme weather event.

Note: The list is not exhaustive, though illustrates the sort of emergencies that can arise in the operation of a Harbour.

## 1.4 Exercising the Plan.

Arrangements will be made to test the Harbour Emergency Response  Plan periodically, preferably by means of a full-scale exercise or a tabletop exercise if more appropriate, proportionate and achievable.

# 2.0 Action

Purpose - The Emergency Response Plan is to ensure the following action is taken:

- Ensuring the appropriate level of emergency services assistance is on site, as and when required.

- Risk assessments are undertaken prior to any action being considered.

- Primarily to minimise injuries/fatalities and thereafter damage to property. It is essential people do not endanger their own safety.

- Minimising risk of further injuries/fatalities, Minimising suffering of people/damage to other property.

- Ensuring other Harbour activities, which may affect, or be affected by the response to an emergency, are aware of the situation and appropriate actions are taken to ensure that activities are stopped or controlled as required.

- Action to prevent’ minor becoming major’ incidents in a timely manner.

- Evacuating people when deemed appropriate, to a “safe distance”, to safeguard life.

- Removing casualties from the scene of the incident to a safe location.

- Provision of first aid and hospitalisation arrangements, including assignment of dedicated first aiders to handle casualties and direct emergency services to the scene of an incident.

- Establishing operational controls and communication channels.

- Control of shipping movements including those perceived to be in danger and if necessary, the closure of the Harbour. This will include designation of certain vessels to assist with the incident.

- Assistance with salvage operations and assisting the emergency services.

- Safeguarding the environment, where possible.

- Facilitation of an investigation into the incident and lessons learnt in order to prevent a future recurrence of a similar loss scenario.

- Restoration of normal operations as quickly as possible.

## 2.1 Types of Potential Incidents

- Collision, Allision, Sinking, Stranding.

- Escape of toxic gases, liquids and hazardous chemicals.

- Explosion.

- Fire.

### 2.1.1 Potential Sources of Incidents

- Commercial vessels.

- Pleasure craft.

- Vehicles operating within the Harbour area.

- Persons in the vicinity of the Harbour

## 2.3 Dangerous Goods

Lynmouth does not handle hazardous cargo in so far as shipping is concerned.

One of the most important things to be established at the start of any incident will be what Dangerous Goods, if any, may affect, or be affected by the incident.  Records of all dangerous goods should be handed over to the senior fire officer, on their arrival on scene.

# 3.0 Raising the Alarm.

In an emergency it is vital the alarm is raised immediately by 999 call to Emergency Services.

All reasonable steps must be taken by those at the scene of an incident to render whatever aid is immediately available.

The decision to designate a Harbour emergency and implement this plan rests with the Town Clerk.  However, a full-scale emergency can only be designated by the emergency services who will make that decision based on consultation at the scene.

All incidents occurring at the Harbour are to be reported to the Town Clerk using all available means in the first instance, The Town Clerk will be responsible for notifying the emergency services, if not already notified.

If the Town Clerk is not contactable then emergency incidents should be reported to the North Devon Council Emergency Officer. NDC will then be responsible for designating a Harbour emergency.

The responsibility for raising the alarm using all available means rests with the Master /Skipper of the vessel/s involved with the emergency or with the person/s who are first on the scene.

## 3.1 Means of Raising the Alarm

- A loud vocal alarm.

- Ring 999 and report exactly what has been seen.

- Advise Council Office landline (working hours only) 01598 752384

- Send someone to the Town Council office or nearest building.

If the Town Clerk is not contactable then emergency incidents should be reported to 999 or the North Devon Council Emergency Officer on 01271 388240 as appropriate.

Due to the size and proximity to the highway of Lynmouth harbour all Serious and Major incidents are to be treated as emergencies and as such they are to be considered a threat to the harbour, other craft, vehicles and people.

In all cases pass details of location, casualties, missing persons and vessels involved.

On arrival the 999 Services will become On Scene Commander and control the emergency response

# 4.0 Incidents within the Jurisdiction

The response plan provides a specified course of action intended to minimise the risk of injuries and / or fatalities.

A stranding, capsizing or sinking may occur because of a number of differing reasons. It is essential given such a circumstance, action be taken as a matter of urgency, to mitigate the worst effects of such an incident.

At all times during the incident response period, the vessel’s Master will retain command of the vessel. The Town Clerk, in consultation NDC EPO and the M.C.A. Counter Pollution and Salvage Officer, if required, will be responsible for any recovery operation.

The Council team will maintain regular contact with the vessel involved in the incident, to ascertain and review any changed requirements and action which needs to be undertaken. They should report any information to the Town Clerk, or the emergency services, upon their arrival.

In anticipation of any pollution around the Harbour, in consultation with the Harbour Users Group a localised pollution response is in being. The Emergency Oil Spill Contingency Plan is held by the Town Clerk. Minor spill equipment will be stored in the Lyn Sailing Club Safety Boat storage facility (opposite the Harbour) in a big yellow bin containing a 120 Spill Kit (Pads, Cushions, Socks, Bags & Ties)

Any incident that results in evacuees, casualties or fatalities will require a co-ordinated response from the emergency services and other responding agencies, who have specific plans for dealing with all eventualities. Such arrangements would be progressed via North Devon Council which would be established at Lynton Town Hall.

## 4.1 Responsibilities’

It must be assumed that should there be an incident, employees would be required to take on certain responsibilities, beyond those as normally expected of them. These roles will be as follows:

Head of Communications: Town Clerk

Head of Incident Control: NDC Emergency Planning Officer

## 4.2 General Guidance Points

The following key points must be considered:

It must be emphasised that, at no time, should any Council employee become or allow any third-party individual(s) to become involved in any action which might endanger their own personal well-being.

Each member of staff will always follow the directions of their immediate supervisor, or if not available, the next person upwards within the chain of command.

Where other third-party individuals become involved in an emergency response, any such individual will always follow the instructions provided to them by the emergency services.

It is understood various members of staff will have been trained as qualified first aiders, or in responding to various styles of incidents. These personnel must be given priority as regards issuing directives to other members of staff.

Any action undertaken in response to an incident must be done, with the involvement / active participation of any relevant parties. This must not mean that decisions are subordinate to any directives given by such organisations.

# 5.0 Fires

The following procedures should be always followed, in relation to all fire incidents:

## 5.1 General

The Devon & Somerset Fire and Rescue Service (DSFRS) should be called with immediate effect.

The priority must be to raise the alarm and evacuate all persons from the area in which a fire is taking place.

The DSFRS Incident Officer will, upon arrival, take command of the scene. They will possess sole responsibility for the incident response, until such time as the situation has been brought under control.

On no account should either the Council team or members of the public consider becoming involved in any substantive fire-fighting measures, unless specifically instructed to do so by the fire and rescue service commander.

## 5.2 Fire on Vessels

For fires on vessels, the firefighting effort will be managed by the Master/Skipper of the vessel until arrival of the Fire and Rescue Service (DSFRS).  On arrival at the scene the Senior DSFRS Officer present will assume control of the incident.

During the incident the Town Clerk and Senior DSFRS Officer should maintain close contact and if there is any concern regarding the safety of the Harbour or the future use of the Harbour arises then a coordinated response will be initiated ensuring the continued safe and efficient running of the Harbour.

In cases of suspected arson or sabotage the Police will liaise directly with the DSFRS.

## 5.3 Fire on Land

For fires originating on the land side of the Harbour the steps detailed in 3.1, Means of Raising the Alarm, should be followed, 5.1, General, should also be followed, if able to do so, and the Fire and Rescue services and Town Clerk called. The senior Officer on site will take control of the incident and keep in close contact with the Town Clerk to coordinate best use of personnel and resources.

## 5.4 Action by vessel

Incidents involving any size of vessel often happen very quickly and can become out of control in a very short time.  Therefore, it is essential any response to such an incident is aggressive and positive.

Having raised the alarm, responsibility for fighting the fire, or dealing with any other emergency, rests with the vessels Master/Skipper until the arrival of the Fire and Rescue Service (DSFRS).  Overall control of the operation will then pass to the DSFRS Senior Officer present, who will follow DSFRS procedures.

A check of all personnel on board must be taken by the Master or senior on-board crew member to account for missing persons.  This should be reported immediately to the DSFRS on their arrival on scene.

Medical Services should be alerted.  The need for ambulances will be ascertained as soon as is practicable, either by the vessel owner, at the initial emergency call, or later by the DSFRS if required.

The DSFRS will set up a control point where contact can be maintained between the DSFRS, other emergency services, vessel owners and the Town Clerk.

The vessel owner remains responsible for keeping the Town Clerk informed of all developments affecting the safety of their and other vessels.

## 5.5 Access to site

Access to the site must be restricted to essential personnel only, as and when sufficient manpower has been brought on site. This should, if necessary, be by blocking off roads and public access to the scene of the incident. The ‘On- Scene Commander’, in conjunction with the emergency services, will be responsible for deciding upon any exclusion zone to be put in place. Should reflective tape be available, this should be used to mark the boundary of the exclusion zone. This should include temporary barriers and fencing. If required, temporary buoys and beacons, advice may be sought from Trinity House for this. Vehicle access routes must be made known to all the emergency services.

### 5.5.1 Personnel on site

Access should only be permitted to persons who have a valid reason to enter the area and are authorised by OSC to do so and then, only once the individuals concerned are made fully aware of the potential hazards involved in any recovery operation and of their duties / designated role(s).

### 5.5.2 Incident Report & Log

It is vital that an Incident Report Form be completed & Log must be started as soon as possible by the Town Clerk, or the person designated by them.

On arrival of the NDC Loggist this responsibility will be passed to them.

The log must make a written record of all actions, decisions and communications taken in relation to an incident. This must be retained in a central location.

Templates for Incident Report Form / Incident log & Resources Used Log

### 5.5.3 Risk assessment.

A risk assessment must be carried out, prior to any response being considered. Where more than 5 people are involved in an operation, this assessment must be in a written format. The risk assessment should be certified by the ‘On Scene Commander’.

It should include an analysis of

1) The tasks(s) to be undertaken,

2) The hazards to be encountered,

3) Control measures required to minimise the risks,

4) Personal Protective Equipment required.

Lynmouth Harbour will utilise the Dynamic Risk Assessment Form ,annex 2.5. Any other Dynamic Risk Assessment Form, produced by the 999 emergency response services may also be used.

### 5.5.4 A site survey analysis.

This should include an analysis of the following factors:

- Communication requirements,

- Layout of the incident area,

- Possible exposure to extremes of temperature,

- Water hazards,

- Feasibility of handrails,

- Rescue craft and ropes and other associated equipment,

- Hazards to hearing, sight and inhalation of toxic substances,

- Shelter from extremes of weather,

- Lighting conditions,

- Machinery usage,

- Manoeuvrability around the site of the incident,

- Restrictions as to control of pedestrians and vehicles in area

- Collection of evidence / samples where appropriate,

- Designated decontamination / treatment areas,

- Facilities for the disposal of contaminants / mortalities,

- Surface terrain and visibility.

## 5.6 Action by the Fire and Rescue Service

All actions will be in accordance with DSFRS Tactical Information Plan. This will include but not limited to;

Establishing liaison with the Town Clerk and the Vessel owner as soon as possible.

The initial response pumps would assemble at a designated point near to the incident.

It is unlikely that an Incident Control Vehicle (ICV) would be called forward for a 2-pump incident.  If 4 pumps were called forward it is likely a Control Point would be set up in the ICV.

A tally system would be established for all personnel boarding a vessel.

Liaison will need to be established to consider aspects of pollution.  The DSFRS, Environmental Agency and the Town Clerk and/or Marine Advisor will coordinate utilising the OSCP.

## 5.7 Action by the police

Initially a Police Liaison Officer will make contact with the Town Clerk/Council.  This will either be in person or by telephone.

The first officers attending the scene should not get involved in rescue work.  They should survey and assess the situation before disseminating the following information to the Police A.C.R. using the pneumonic ‘METHANE’: M/ETHANE - JESIP Website

M – MAJOR INCIDENT DECLARED?

E – EXACT LOCATION.

T – TYPE OF INCIDENT.

H – HAZARDS PRESENT, POTENTIAL OR SUSPECTED.

A – ACCESS (SAFE ROUTES IN/OUT).

N – NUMBER/TYPE/SEVERITY OF CAUSALITIES.

E – EMERGENCY SERVICES PRESENT/REQUIRED.

*The officers should then commence a log.

On receipt of an emergency call the Police will assume responsibility for cordons, security, traffic management and evacuation.

As soon as possible after the declaration of a major incident the Police will establish an Incident Unit and maintain contact with the Medical Officer in Charge, DSFRS, Town Clerk and/or Marine Advisor, on scene Control Point, HM Coastguard (Falmouth) and Local Authority.

M/THANE form

### 5.7.1 Inner Cordon

Depending on the nature and location of an incident, it may be necessary to establish inner and outer cordons. the Fire and Rescue Service are responsible for establishing, securing and managing the inner cordon and are responsible for safety management of all personnel within the inner cordon. Only authorised personnel who have a role, are suitably briefed and are wearing appropriate protective clothing will be permitted entry.

Briefing should include information on hazards and any evacuation signal.

The Fire and Rescue Service will log and verify their own service personnel and other agency staff entering the inner cordon.

Employees of partner agencies and the Fire and Rescue Service also have a legal responsibility to follow their employer’s guidance and to look after their own safety and the safety of others under the Health and Safety at Work etc. Act 1974, Section 7.

### 5.7.2 Outer Cordon

The Police will control all access and egress points to the outer cordon. The identity of non-emergency service personnel requiring access through the outer cordon will be checked and recorded by the Police.

### 5.7.3 Operations Analysis

A review should be made as to what equipment is available to assist in any operation: radios, fire extinguishers, stretchers, lighting, hoses and pumps, temporary disposal drums / sites, motor and commercial vehicles, buses, cranes, lifejackets, cooking facilities, winches, air quality measurement devices, helicopters, tugs and other privately-owned craft and pollution control equipment. Dispersants can only be used with the permission of the MCA.

## 5.8 Action by Coastguard

If the Coastguard is the primary contact, they will initiate the following actions:

•	Alert emergency services as required.

•	Send units to assist with search for survivors.

•	Despatch a mobile unit to the Harbour to assist/advise on care of personnel.

•	Alert the Town Clerk/Council as soon as is reasonably practicable.

Communications will be established and maintained on internal emergency service frequencies.

# 6.0 Medical Emergencies

In the event of a medical emergency on Harbour land the initial response is to raise the alarm as per section 2.3 and to call for an ambulance. There are 3 AED defibrillators available at the following locations

- Flood Memorial Hall: WTW  ///stilted.steeped.patch

- Cliff Railway: WTW ///nasal.name.canoe

- Lyndale Tea Rooms: WTW ///punctual.coping.hamster

## 6.1 Medical Emergencies on board

Medical assistance can be divided into two separate categories:

- Vessels in Harbour at anchor or underway.

- Vessels at Berths Alongside.

Requests for routine medical assistance should be made by the quickest available means, normally V.H.F. radio or mobile phone.

Requests for urgent medical assistance received from a ship in the Harbour should be relayed by the recipient to the ambulance service.

Requests for urgent medical assistance received from a vessel at sea should be directed to Falmouth Coastguard (M.C.A.) who shall make the necessary medical arrangements.

Emergency Rest Centres, of which there are 5 designated in North Devon, are established by Devon County Council (D.C.C.).

Operational Emergency Rest Centres are managed by Devon Social Services, in conjunction with the appropriate District Council.

## 6.2 Action by Medical Services

Medical Services will respond to the information given in an emergency call.  It is normal for a Paramedic Crew and an Ambulance Incident Commander (AIC) to attend in the first instance.  If a Major Incident is called the AIC will have overall responsibility for the activity of all NHS personnel at the scene.  A Medical Incident Officer may be appointed from the cadre of MIMMS trained doctors and will be responsible for the management and deployment of medics and nurses.  They would work closely with the AIC.

For a major incident a Control Point would be established and this would become the ambulance rendezvous point.  From there they would be despatched to their required destination.

Current SWAST policy states that its personnel will not deploy beyond the shoreline.  Therefore, it is likely that casualties would be transported ashore by Helicopter and/or Lifeboat.  The casualty landing site for the Lifeboat is dependent on tidal and weather constraints.

## 6.3 Helicopter Operations & Landing Sites

Helicopter landing operations generally use Manor Foreshore or Manor Green.  999 control know the key code for the key box fixed to the wall of Rock House Hotel so as to open the gates to the Manor area.

Any landing site should be as level as possible, free of any style of obstruction and large enough to permit manoeuvring.

The landing area should be completely free of vehicles, people, animals, and particularly all loose items / debris which could otherwise be sucked into the aircraft’s engines. If time permits, a sweep should be made of the area for any materials which may cause problems.

### 6.3.1 Night operations.

Night-time flying, especially landing, is a harder task for aircrews, largely due to the absence of visual reference points.  For safety reasons, a larger landing site is needed, than in daylight hours. The other criteria are broadly similar.

The landing area should be lit by vehicle headlights, from the extreme edge of the perimeter. It is important that vehicles are not so tasked, extinguish their lights and stay clear of the landing area.

If, in the opinion of the helicopter commander, conditions prevent a safe landing at the Harbour, an alternative location shall be decided in consultation between the helicopter pilot / M.C.A. / the Police and the Council.  An approved Night landing site is Holman Park in Lynton.

Current SWAST. policy states its personnel will not deploy beyond the shoreline. Therefore, it is likely casualties would be transported ashore by Helicopter and / or Lifeboat. The casualty landing site for the Lifeboat is dependent on tidal and weather constraints. The casualty landing site for Helicopters would be the the Manor area, if sufficiently clear, or alternatively on Holman Park.

# 7.0 Evacuation Procedures

The Incident supervisor shall nominate the appropriate assembly point, which may be dictated by the time of day that an incident occurs, the prevailing wind and weather conditions, what other operations are taking place and the nature of the incident.

This plan is aimed at initial response to incidents and the evacuation of personnel to a safe place within the Harbour area.

Where many evacuees require longer term shelter and / or humanitarian assistance, this shall be arranged as a co-ordinated action with the Town Clerk/NDC EPO

Any evacuation will be controlled and directed by the Incident Commander in consultation with the emergency services.  999 teams shall commence evacuation procedures, potentially assisted by the Police when sufficient personnel are available.

Parking space must be cleared at Evacuation Point(s) to facilitate any required measures.

Any evacuation must be carried out in an orderly fashion.

Any properties / areas as evacuated should be clearly marked, to assist with the operation.

IMPORTANT: It is essential that any evacuation site as chosen should be up-wind from the source of any leak, spill, smoke or fumes.

# 8.0 Subsidiary Considerations:

Note: It will be imperative the Harbour’s insurers are kept appraised as regards investigations into the fire. Though they may rely on the fire and rescue service / police report(s) to assist in reviewing any causation process, they may well decide to appoint their own expert, in order to review the findings of the fire & rescue service and police. Evidence as to cause should therefore not be tampered with until such time as insurers have had the opportunity of carrying out their own investigations.

The Senior Fire Service Officer present will control fire-fighting operations, both on and offshore.

Consideration should be given to bringing on site other response agency vehicles, if available.

If a situation deteriorates to such an extent that further action is deemed detrimental to the general well-being of the response team, the Senior Fire Officer will possess the right to order that any rescue operation cease.  Should the incident involve a vessel in distress, any decision in this regard will be conveyed to the vessel master and Council at the earliest opportunity.

Closure of the Harbour, control of shipping movements, or the creation of exclusion zones will be the responsibility of the Town Clerk in consultation with NDC and the Marine Advisor.

It needs to be considered that V.H.F. is a completely open communications system and as such any information that is broadcast is uncontrolled. Most commercial vessels carry alternative communications devices such as mobile phones and email.

A Ship’s Master who considers his vessel (other than the incident vessel) to be endangered must only take the action he considers necessary for the safety of his own ship.

Any vessel involved in an incident within the Harbour shall only be moved with the express permission of the Incident Commander, Town Clerk and/or the Marine Advisor.

Vessel incidents will be monitored by the Maritime and Coastguard Agency. If a vessel has declared a maritime emergency, the Coastguard will be responsible for co-ordinating any response offshore.

Until such time as any such vessel enters the Harbour’s confines, the M.C.A. / Coastguard will have sole responsibility for the movement of any such vessel.  The Council may refuse access to the Harbour to a vessel, which is in a dangerous condition.  The vessel may be directed to a safe anchorage, if weather permits. S.O.S.R.E.P. may over-rule the Council and insist the vessel is accepted into the Harbour. In this instance, S.O.S.R.E.P.’ S ruling must be adhered to.

The Police will take responsibility for control of the scene of the incident and protection of any crime-scene.  They will also be responsible for the identification of any fatalities and injured personnel, the provision of adequate records and the advising of relatives. The locus of any incident may be designated as a crime scene by the Police once the area is safe to enter and the incident has been brought under control.

If a vessel has the capability of assisting with the incident, i.e. firefighting / towing / rescuing personnel and it is deemed safe to enter the Harbour area, the Incident Commander may consider asking such a vessel for assistance.

# 9.0 Pollution

Multiple factors have been taken into account when considering the effects of a pollution incident in Lynmouth Harbour.

Lynmouth Harbour has a minor Oil Spill Contingency response plan which covers local procedures for any minor incident which results in pollution occurring within the Harbour Area or Limits.

This will involve liaison with mooring owners who have experience dealing with such incidents as part of their occupations etc.

Where the incident can be managed locally by these local experts and Council staff using the Yellow Box Minor Spill kit in the LSC Safety Boat store then this will happen.

Where the spillage is more than this can address then North Devon Council Emergency plan will be instigated and Ambipar, under contract to NDC will be called to deal professionally with the situation and resolve the issue.

## 9.1 Actions by Vessels

It is the duty of the Master/Skipper of every vessel in the Harbour to ensure every step is taken to prevent pollution entering the Harbour from his vessel.  This includes all leisure craft.

In the event of pollution, immediate action must be taken to restrict further pollutant entering the water.

### 9.1.1 Reporting of pollution

The responsibility for reporting a pollution incident rests with the vessels Master/Skipper in all cases.

All persons seeing pollution should report it immediately, it is better to have had an incident reported twice than to not have had it reported at all.

### 9.1.2 Method of reporting

#### 9.1.2.1 Vessels

Vessels sighting pollution should:

Inform Lynton & Lynmouth Town Council and pass information stating where, what and how much pollutant has entered the water and the immediate action taken.

If the Council cannot be immediately contacted: Call ‘Falmouth Coastguard’ on VHF Ch. 16 requesting that the information be passed to the Harbour Authority via landline.

#### 9.1.2.2 Persons Ashore

Persons sighting pollution from ashore should contact the Council by all available means.  If the Council cannot be reached immediately then Falmouth Coastguard should be contacted.

Note: In all cases of pollution, a CG77 POLREP is required to be submitted to Falmouth Coastguard.

## 9.2 Actions on receipt of Pollution Report

### 9.2.1 Town Clerk

- Immediately investigate report.

- Deploy Tier 1 response equipment and, if the incident is significant, activate the NDC Emergency Officer contact

### 9.2.2 Shore Operator

- Immediately isolate the source of pollution and take all steps to stop the pollutant entering the harbour.

- Inform Council.

- Activate own emergency procedures.

- Assist Council in the clean-up operation.

### 9.2.3 Vessel

- Take all steps to stop further pollutant entering the harbour.  Activate damage control procedures (i.e. Shipboard Marine Pollution Emergency Plan, SOPEP) if necessary, blocking scuppers etc.

- Deploy catchment ropes/booms etc.

- Stop cargo operations and take steps to ensure that fire dangers are not increased.

## 9.3 Containment of Pollution

Lynmouth Harbour area contains a wide variety of wildlife flora and fauna.  Therefore, it is policy to make every attempt to contain any oil spillage close to its source and/or in areas of natural collection within the Harbour.  It is essential that early action be taken to contain an incident.

## 9.4 Damaged/Leaking vessel wishing to enter Harbour

Vessels who require entry into the Harbour, that are leaking or might leak oil as a result of damage, must seek the express permission to enter from the Council.  Until this has been granted the vessel will remain outside of the Harbour limits.

Before granting permission to enter the Harbour the Town Clerk will require confirmation that the leakage has stopped.  They will seek the advice of the Marine Advisor, Pollution Officers from County and District Councils and the MCA’s Counter Pollution and Salvage Officer. (CPSO).

On arrival into the Harbour the vessel will require immediate survey to ensure that further pollution is not occurring nor is likely to occur.  A strict and continuous watch should be set up to monitor for leakage for the duration of the incident.

If the vessel is leaking or damage is such that leakage is unavoidable permission will not be granted by the Council unless the Secretary of State’s Representative (SOSREP) issues a formal Direction to the Town Clerk and to the vessel.

## 9.5 Pollution entering lYNMOUTH Harbour from seaward

On receiving the initial report of pollution that is likely to enter the harbour the Town Clerk will evaluate the situation and dependant on this activate either Tier 1 or Tier 2 response levels as detailed in the OSCP.

Every effort will be made to prevent the movement of pollution from seaward into the Harbour.

# 10.0 Recovery & Salvage Operations

Should a vessel be in danger of stranding, capsizing or sinking, the Town Clerk may request assistance from other vessels in attempting to move the vessel to a safe location.

Ambipar should be contacted, in order that the latter can aid where deemed appropriate.

Severn Seas Shipping, a local salvage operator, should be contacted, in order that the latter can aid where deemed appropriate. 01598753988

Given the circumstances that several vessels are brought in to assist in the handling of an incident, all communications should be restricted to the Emergency V.H.F. channel 16 only.  Such communication must be kept to a minimum, to avoid the radio channel becoming blocked. Such communications as made, should be ‘clear and concise’.

The Town Clerk and/or Marine Advsior will be responsible for the co-ordination of assistance as may be provided by Harbour users capable of aiding.

If a vessel capsizes within the Harbour or on its approaches, only limited assistance can be provided. Given such a situation, it would be imperative to bring divers on-site, who could not only assess the condition of the vessel involved in the incident though also work out as to what style of remedial action might be necessitated. Possible local diving companies include:

Easy Divers – 07833020424

Lundy Diving – 07971462024

Ilfracombe and North Devon Sub Aqua – 01271855652

In circumstances where a vessel has capsized outside of the Harbour Limits, the rescue operation shall be under the supervision of Falmouth Coastguard (M.C.A.).

It must be noted that action taken to tow a capsized vessel may result in it sinking. Should any consideration be given to the possibility of towing the vessel away from the main areas of the Harbour, it would be wise to consider seeking guidance from Ambipar in the first instance. Any tow lines as may be attached in such circumstances must possess release mechanisms, in order that should the vessel move out of control, it can be released, and further casualties are avoided.

Any action as undertaken should act to ensure wherever possible that access to the Harbour whether by vehicles or by vessels is kept clear.

Given the situation whereby a large vessel becomes stranded or grounded on the shore, or in the Harbour approaches, the S.O.S.R.E.P. representative will be the person who needs to be consulted relative to attempts to pull the vessel off.

It may be necessary for a survey of the vessel to be carried out, prior to any movement being considered. It may be best practice to consider leaving the vessel in place until such time as a reputable salvage company arrives on the scene.  Ambipar can help in this regard. It should, however, be borne in mind that if the grounding has taken place on an ebb tide, that the vessel may refloat itself on the next rising tide.  The ship’s master should confirm their tank soundings to determine if there is an ingress of water. Appropriate measures should be considered to prepare for any potential pollution, which might arise, should the vessel refloat. This may involve activating Lynmouth Harbours 2P Oil Spill Responders and deploying booms around the vessel. 5 x 15 mtr Harbo Booms are held at Ilfracombe Harbour.

Booms can be on-scene within 1 hour.

Should a crane be required, there are 2 local haulage companies that have resources to respond to all but a major incident.

In an emergency, a crane could normally be mobilised and on-scene within 1 to 2 hours.

In any circumstance whereby a major salvage operation is considered, the S.O.S.R.E.P. must be contacted prior to any works being commenced, in order that a designated representative can be appointed to supervise any operation. It may be deemed appropriate by the Secretary of State’s Office, to set up a S.C.U. (Salvage Control Unit). This will comprise a salvage company, the Marine Advisor, a ship owner’s representative, Environment Agency personnel and potentially S.O.S.R.E.P. / an M.C.A. Counter Pollution and Salvage Officer.

# 11.0 Personal Protective Equipment (P.P.E.)

The level of Personal Protective Equipment (P.P.E.) should match with the requirements of the task to be undertaken. This may include protective / fire retardant clothing, harnesses, breathing apparatus, floatation suits, gloves, suitable protective eye wear, hard hats, insulated clothing where appropriate, suitable footwear and lifejackets. It might be necessary to consider provision of shelter, accommodation, warm clothing, food, drinks, rest areas, sanitation and first aid facilities on site.

A separate decontamination / storage zone may need to be set up, where contaminated items or serious casualties can be handled. Any effluent created by pollution cleansing activities, should be stored in the same manner as any contaminants and an appropriate waste disposal contractor contacted to arrange for uplift of the materials.

# 12.0 Bodies in the Harbour

In the event of bodies being found in the Harbour or at the scene of an emergency the Police must be informed.

# 13.0 Animals

If animals have to be landed as the result of an emergency or have been illegally landed then the animals would need to be held within a secure area until the matter has been dealt with by Devon County Council’s Trading Standards Service. In the event of the animals being deceased then there is a separate procedure as outlined in the NDC policy.

Procedures on removing dead animals | North Devon Council

# 14.0 On scene management

To achieve a combined and co-ordinated response the capabilities of the emergency services should be closely linked with those of local authorities and other agencies through integrated emergency management arrangements.  The adoption of the nationally agreed structure enables all parties to understand their role in the combined response.

## 14.1 Transfer of Command Post

After initial response and if an incident is declared as a major incident, a central command post will be established.  For a major pollution incident this will be the North Devon Council’s Offices at BEC.  For other emergencies the emergency services will establish a Forward Command Post (FCP)/ E.R.C.C on site.

If a major incident were to be declared then the Devon, Cornwall and Isles of Scilly local Resilience Forum would normally establish a Strategic Co-ordinating Group (a gold level of Command and Control) to run concurrent with the Police Gold Command.

## 14.2 Emergency Response Control Centre (E.R.C.C.)

If the incident is of sufficient scale to merit additional tactical support, Silver Command will be established at Lynton Town Hall. However, if the incident magnitude needs greater resource North Devon Council’s Silver Control suite at Brynsworthy Environment Centre (BEC) will be used.

There will be ongoing communication between the E.R.C.C and BEC throughout the response phase of the incident.

Upon choice of E.R.C.C., the following procedures will be put into place:

- That the following will be notified with immediate effect of the location of the E.R.C.C. and contact numbers for key members of staff:

- The Police.

- The Ambulance Service.

- The Fire and Rescue Service.

- Harbour Users (operating within the Harbour at the time).

- North Devon Resilience.

- Maritime Coastguard Agency.

- E.A.

- Natural England.

- Other emergency plan holders should be contacted, as and when the need arises.

- A command post to be set up at the scene of the incident. The Command Post should be upwind of relevant incidents where smoke / flames /gas etc. are present.

- Specified areas of responsibility must be defined with a section head confirmed, for dealing with the following aspects:

- Injuries and mortalities. Casualties will be the responsibility of the Ambulance Service and fatalities will be dealt with by the Police.

- Physical damage to property, which includes any vessel within the Harbour’s confines.

- Evacuation procedures.

- Pollution and associated clean up measures.

The Centre will be always manned by a member from each of the response agencies involved in the incident.

If required, consideration should be given to setting up a Casualty Clearing Station to remove casualties to a temporary sheltered place of refuge for assessment.  The location may depend on the location of the incident. The Marine Advisor will be aware of what buildings would be available for use.  Advice shall then be obtained from the ambulance service as to where casualties and evacuees shall be transported for the longer term.

A command structure will be agreed to confirm each agency’s responsibility.

All decisions relative to the emergency services’ response will be co-ordinated / controlled from the Centre in liaison with the Marine Advisor.

Co-ordination with the Local Resilience Forum plan and guidance will be the responsibility of the North Devon Council Emergency Planning Officer.

A dedicated mobile number will be assigned within the E.R.C.C., so that action can be co-ordinated.

Any decisions made shall be subject to review with the relevant “section head”. The On-Scene Commander’s decisions will be binding on all others.

The E.R.C.C. will remain operationally in control until such time as it has been agreed by “all” the participating parties, that the “emergency” situation has been brought under control and the “On-Scene Commander’’ has made an official statement to this effect. The response shall then be stood down, and normal operating procedures may resume, if appropriate.

NDC Emergency Team shall maintain a log of communications, discussions held, decisions made, and action taken in response to an incident.  The person making the entry should make short, detailed entries, with timing carefully recorded and if possible, confirm Log Entries with participants.

All external communications will be co-ordinated by the Police with assistance from those agencies represented in the E.R.C.C. and individual staff of all agencies shall be advised that all enquiries must be directed to the E.R.C.C. The Police may set up a dedicated phone number for enquiries in the event of a major incident.

Stakeholders will be requested to attend the Centre, as and when required, in order that they can be provided with designated roles relative to the incident response.

The E.R.C.C. will act to control all shipping movements within the Harbour Limits. This will, if deemed appropriate, include the closure of the Harbour and designation of vessels to assist with any response action. The Marine Advisor has the statutory power to give instructions to vessels, where necessary.

Should the scenario arise that the E.R.C.C. must be located within an alternative building, runners will be employed as relays between the incident scene and the Centre, to assist with co-ordination of the response.

The E.R.C.C. will provide the framework for co-ordinating the action of and liaising with the following:

- The Fire and Rescue Service.

- The Ambulance Service.

- The Police.

- North Devon Council Emergency Planning Officer.

- R.N.L.I.

- M.C.A.

The E.R.C.C. will determine, in the light of available information, which adjacent premises and areas need to be evacuated or alerted.

On discussion with senior personnel from the emergency services and operators within the Harbour, the Marine Advisor will set an appropriate exclusion zone.

Premises to be evacuated / cleared must each be thoroughly checked, to ensure that they are evacuated.  A large “X” in chalk / marker pen / paint, etc. should be conspicuously displayed on or near all entrances / exits of each building, to show that the property is empty, and doors should then, if possible, be locked.

Non-emergency personnel should be prevented from re-entering the area until it is safe to do so. In this regard, it should be noted that stakeholders may possess their own evacuation plans in respect of their premises.

## 14.3 Environment Agency (E.A.)

The E.A. is responsible for environmental protection in England and adopts an integrated approach to the protection and enhancement of water, air and land and associated natural resources.

During an emergency the E.A. will deploy its comprehensive scientific capability to give support and advice to other agencies and to the public on such matters.

In responding E.A. will:

- Deploy appropriate staff to meet local co-ordination arrangements.

- Provide advice on all aspects on environmental impact, protection and recovery.

- Assist in determining the footprint and movement of any contamination.

- Give advice about implications to the environment, containment, storage, transportation and disposal of contaminated liquid or solid waste.

- Maintain operational links with Southwest Water, Local Authorities, Environmental Health Departments and Health and Safety Executive.

Additionally, E.A. has powers to prevent, minimize or reduce pollution of the environment and enforces environmental legislation.

The E.A. performs the following functions:

- Regulates the treatment, storage, movement and disposal of waste.

- Provides, as flood warning authority, regularly updated information on flood warnings (Flood line Alerts) across England.

- Administers jointly with the Health and Safety Executive the Control of Major Accident Hazards (C.O.M.A.H.) legislation.

- Regulates the disposal of radioactive waste and manages English interests in the Radioactive Incident Monitoring Network (R.I.M.N.E.T.).

## 14.4 Secretary of State’s Representative (S.O.S.R.E.P.)

S.O.S.R.E.P. represents the Secretary of State for the Department for Transport. S.O.S.R.E.P. if required, is empowered to make crucial decisions, often under time pressure, without recourse to any higher authority.

S.O.S.R.E.P. will have ultimate responsibility for any marine incident that poses a significant threat of pollution or risk to life.

In the unusual circumstance that directives are issued by S.O.S.R.E.P., they must be always followed. Failure of any individual concerned to follow directions as laid down by S.O.S.R.E.P. constitutes a criminal offence. Should it be deemed necessary, a Salvage Control Unit will be created, to supervise any salvage operation. Such a unit will include the following personnel: a salvage manager from the appointed salvage company, the Marine Advisor, a representative of the vessel owner, an environmental liaison officer and a representative of the Counter Pollution and Salvage Team, nominated by S.O.S.R.E.P.

## 14.5 Emergency Response Centres

The Devon County Council Emergency Planning Service assists the Police at the following emergency response centres.

- Survivor Reception Centres.

- Friends’ and Relatives’ Reception Centres.

- Emergency Mortuaries.

- Evacuation Assembly Points.

- Emergency Rest Centres.

Most Centres listed above are established and managed by the Police.  However, Emergency Rest Centres, of which there are 5 designated in North Devon, are established by Devon County Council (DCC).  Operational Emergency Rest Centres are managed by Devon Social Services, in conjunction with the appropriate District Council.

## 14.6	Support Services

The British Red Cross Society are available on activation of the Devon County Emergency Plan.  They can provide manning and resources for Staff Rest Centres.  They can also provide an ambulance service.

The Salvation Army, who are not primarily an emergency disaster relief organisation, have purpose built mobile units that are stocked and equipped with the resources required for a major incident.  They have supplies of food, blankets and first aid equipment as well as safety equipment, which allows them to become self-contained when on site.   They become activated via the Devon County Council Emergency Response Plan.

# 15.0 Key Considerations

## 15.1 Media & Communications

In the early stages of an emergency the Town Clerk will endeavour to keep the media updated and designate an area for the media in the safe zone.

On arrival of an NDC Communication Officer the Town Clerk will be relieved of this function but continue to liaise closely with the comms officer to enable regular briefings to the media.

Co-ordination with the Local Resilience Forum plan and guidance will be the responsibility of the NDC Communications Officer.

It is understood that a media statement shall be prepared by the Police relative to a major incident occurring within the Harbour area.  The Maritime Coastguard Agency (M.C.A.) may also prepare a statement if the incident is marine related. If it is considered appropriate for North Devon Council to make a press announcement, an “Initial Press Notification” form is attached at Appendix 6.  This shall only be completed and released on the authority of the Town Clerk and/or Marine Advisor following consultation with the NDC EPO, The Police and other agency leads that are present if so required.

Following an incident, there will be heightened interest shown by the local and national media. In order that enquiries are handled in an efficient and timely manner, the following procedures must be always followed:

All enquiries of whatsoever nature, whether from local or national media, must be referred directly to the Town Clerk and/or Marine Advisor or in their absence, the North Devon Council section head.

The Marine Advisor / Council section leader shall decide as to whether a response to any enquiry would better be provided by either the Council, the Police or stakeholder organisations. If any Lynmouth Harbour response is considered appropriate, an “Initial Press Notification”.

When considering issuing a formal response, the following must be considered:

- A proper investigation of the circumstances surrounding an incident must be carried out, in order that an accurate assessment can be provided to any third party.

- Only factual information needs to be disseminated.

- The role of the emergency services in dealing with the incident and any ensuing casualties.

- Emphasis being given on protecting physical assets, people and the environment.

In the event of a major incident, the Public Communications Group co-ordinated by the Police, will issue press releases relating to the incident.

Enquiries from the general public will be handled in a similar manner as that from the Media. Any communication must be referred to the Town Clerk and/or Marine Advisor, or in their absence, the Council section head for their respective comment. Details of any such communication must be recorded for further reference, should the need arise.

If a media representative presents themselves in person at the Council Office, they will be handled with due diplomacy and be requested to wait until such time as the person in charge is present to meet them. In no circumstance, shall any issue be reviewed with the media representative, until such time as due authorisation is provided. Any such person must not be given access to any of the Harbour’s premises.

All persons involved with any incident shall be requested to refrain from making comments relating to any ongoing incident on social media.

It would be envisaged that a dedicated telephone line within the E.R.C.C would be set up to handle media enquiries. The identity of this number must be made known to all stakeholders and public authorities.

## 15.2 Means of communication.

During an emergency the Emergency Services will use their own internal radio frequencies for inter-agency communications.

### 15.2.1 Mobile phone communication.

Where possible, communications should be made by mobile phone in order to keep other channels clear. It is essential though, that calls are restricted to essential communications only, as otherwise, the available network(s) will reach their capacity limits very quickly and might even collapse.

By posting one officer from each of the response agencies within the E.R.C.C., enables effective communications can take place between all of the emergency services, which will be operating on other frequencies.

## 15.3 Contact with Key Service Providers

The key service providers must be notified of any incident as soon as possible. They must be kept abreast of key developments.

These are subdivided as follows:

### 15.3.1 Insurance

It is essential in any such situation to keep the Harbour’s insurers closely advised of developments as regards any incident and if possible, seek their prior agreement to ‘any’ intended course of action.

This would involve the following procedure:

Notification of the incident being given to the Harbour’s insurance brokers with immediate effect.

Should a surveyor, loss adjuster and / or lawyer be appointed, that sufficient information be supplied to them at the earliest possible stage. Prior to any information being passed on, prior approval must be obtained from the Chief Executive. Any information as proposed to be released, must be “screened” in the first instance by the “On Scene Commander”, in order that only suitable information is divulged to them.

Photographic and video / C.C.T.V. records should be kept, wherever and whenever possible. This will assist in any ensuing investigations into the cause and effect of the incident. Any suggested course of action involving the involvement of third parties such as insurers must be reviewed with the Marine Advisor or the Duty Marine Advisor, prior to any action being taken.

Should it be deemed appropriate, a suitably competent lawyer should be instructed to protect the Harbour’s interests at the earliest opportunity, over and above that as may be instructed by the Harbour’s insurers. It would be envisaged that the latter would work in conjunction with the Marine Advisor. Insurers must be kept updated with regular bulletins as regards developments in relation to the incident. This should be daily, if circumstances warrant such action.

Should major expenditure be envisaged, a request should be made at the earliest available opportunity, for an interim / advance payment from insurers, in order to ensure stability of cash flow and to cover the cost of any potentially substantial remedial works.

Should it prove necessary, arrangements should be put in place for a meeting to be held between the Town Clerk and insurers, to seek agreement on the proposed response to the incident. Should time not permit, insurers advice should be sought by calling the broker in the first instance or if deemed necessary, their office directly.

An active dialogue with the Harbour’s insurers should continue until such time as they have agreed to meet their obligations in full and without any caveat being provided as regards payments / proposed course of action.

It is essential that the prior approval of the Harbour’s insurers be obtained wherever possible, as regards any proposed course of action. Should this not be possible, any proposed action must and only be sanctioned by either the head of section of the Council or in the absence of both, the Duty Marine Advisor.

### 15.3.2 Harbour Users

It is essential that Harbour users are kept appraised as regards any incident. The following steps must be undertaken:

Notify Harbour users as and when required. Some relevant Harbour users may be asked for assistance by supplying personnel or equipment where necessary.

Advise Harbour users of any implications the incident may have as regards their ability to operate within the Harbour. This must include a request to suspend services until such time as the incident has been brought under control.

The Harbour users should receive regular updates as regards the Harbour’s operational status.

The Harbour users must be advised as and when the incident is brought under complete control, in order that normal port operations are resumed.

### 15.3.3 Government Authorities

It is important that the Local Authority and Government Agencies including the M.C.A., Marine Accident Investigation Branch (M.A.I.B.) (if the incident has involved a vessel) and the Health and Safety Executive (H.S.E.) are notified of any incident at the very earliest stage and are kept fully and regularly appraised of any developments in relation to the incident.

The Town Clerk and/or Marine Advisor shall be responsible for contacting Falmouth Coastguard and M.A.I.B.

It is essential the following steps be taken in this regard:

That any communication be channelled through the Marine Advisor.

That a copy of the relevant legislation be always kept in an easily accessible location within the E.R.C.C. and that it is referred to, prior to any substantive course of action being undertaken.

That the instructions as given by such bodies are always complied with. Should any confusing or contradictory advice be given, such concerns must be raised with the Marine Advisor or in their absence, the Duty Marine Advisor, prior to any action being considered.

The Authorities are duly advised when the “stand down” order has been given.

# Annexes

## 1.0 Contact Directory

| Lynmouth Harbour |  |  |
| --- | --- | --- |
| Ilfracombe Harbour Office | 01271862108 |
| Harbourmaster mobile | 07775532606 |
| Deputy Marine Advisor mobile | 07748111290 |
| Town Council Offices | 01598752384 |
| PMSC - Designated Person (Jon Triggs) | 01271388221 |
| Devon County Council - Main | 01392382680 |
| North Devon Council Emergency Customer Services Emergency Planning Officer | 01271388240 01271327711 07584382324 |
| Resilience & Emergencies Division 24hr Emergency | 03034442718 03034442799 |

| Emergency Services |  |
| --- | --- |
| Fire and Rescue Service. | 999 01392872200 |
| Police. | Emergency 999 Non-emergency 112 |
| Ambulance | 999 |
| HM Coastguard – Falmouth Operations Centre | 999 01326317575 |
| R.N.L.I. (Ilfracombe Lifeboat House). | 01271863771 |
| Ambipar Response Ltd 24hr Emergency | 01202653558 |
| Government Agencies |  |
| Health & Safety Executive. | Online at www.HSE.gov.uk/ |
| Environment Agency | 0800807060 |
| Resilience & Emergencies Division 24hr Emergency | 030 344 42718 030 344 42799 |
| Marine Management Organisation (MMO) General Enquiries Duty Officer | 03001231032 07770977825 |
| Natural England | 03000601200 |
| D.E.F.R.A. Duty Room | 03450518486 |
| INSURERS |  |
| Johnathon Meiseles Zurich. | 07971009498 |
| OTHER AGENCIES |  |
| Energy Supplier. EDF | Electric 105 Gas 0800 111 999 |
| British Red Cross Society Crisis response contact centre. | 07734734342 |
| Salvation Army (Emergency response). | 07711731751 |
| St Johns Ambulance (24 hours) | 03300535097 (Select option 1) |
| Devon Wildlife Trust, Exeter | 01392 279244 (Use answer phone) |
| RSPB, Southwest England | 07899 797473 |
| RSPCA | 03001 234999 |

Key

## 2.0 Templates

### 2.1 Incident Report Form

| Casualty Details: | Comments: |
| --- | --- |
| Vessel Name / Location: |  |
| Length Overall, Tonnage and Draught. |  |
| Contact Details: |  |
| Details Of Incident Including Extent of Casualties: |  |
| Casualty Situation: |  |
| Type Of Cargo and Stowage. If Oil Spill, Presumed Type and Rate: |  |
| Number Of People Aboard: |  |
| Any Potential Language Barrier: |  |
| Potential Hazards / Avoidance Techniques Possible Such as Towage: |  |
| Prevailing Wind and Weather Conditions: |  |

### 2.2 Incident Log

| Incident. |  |  | Date of Incident. |  | Page No. |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date. | Time. | Comment / Action / Details. |  |  |  |  |

### 2.3 Resources used in Incident response Form

| Date. | IHT |  |  |  | External Contractors. |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Personnel / Hours. | Materials. | Boats. | Other. | Personnel. | Booms. | Other. |

### 2.4 Press Release Form

#### 2.4.1 Initial Press Notification.

Lynmouth Harbour confirms that an incident has occurred (Give brief description of incident) at approximately (Insert time and day).

Emergency response procedures have been initiated, and the relevant authorities have been advised. All support services are being coordinated through the Council’s Incident Response Team and every possible effort is being made both to minimise risk to personnel at the scene and to contain / mitigate any adverse effects.

Further information will be released, at a press conference scheduled for…………………

Authorised Signatory…………………………………………………………………………………………….

Position………………………………………………………………………………………………………………..

Lynmouth Harbour.

Website: www.lyntonandlynmouthtowncouncil.gov.uk/lynmouth-harbour/

E-mail: kevin.harris@northdevon.gov.uk

### 2.5 Dynamic Risk Assessment Form

“Do Not Carry Out Any Task That Is Not Safe”

|  | Amend existing risk assessment |  | Risk assessment appending # : | RA/….…. |
| --- | --- | --- | --- | --- |
|  | New Task (send copy to Marine Advisor) |  |  |  |

Section 1 - Job

|  |  |  | Division: |  |  |  | Marine |  | Harbour |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Job: |  |  |  |  |  |  |  |  |  |  |  |
| Location: |  |  |  |  |  | Date: |  |  |  |  |  |

Section 2 – Hazards  Tick relevant box(es)

|  | Access and egress |  | Electricity |  | Falling Objects |  | Vibration/ Noise |  | Drowning |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Slips, trips, falls |  | Confined Spaces |  | Dust/Vapour/Fume |  | Hazardous Substances |  | Radiation |
|  | Working at height |  | Unguarded Machine |  | Moving Objects |  | Weather Conditions |  | Asbestos |
|  | Lone Working |  | Manual Handling |  | Vehicular Movement |  | Collapse |  | Working near water |
|  | Collision |  | Grounding |  | Sinking |  | Fire |  | Explosion |
|  | Other: |  |  |  |  |  |  |  |  |

Section 3 -  Risk Controls

| Hazard |  | Who could be harmed | Control Measures | Actioned |  | Level of risk H/M/L |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Yes | No |  |
| 1 |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |

Section 4 – Personal Protective Equipment Requirements  Tick relevant box(es)

|  | Head Protection |  | Gloves |  | Fall Protection |  | Other (specify) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | High Visibility Garment |  | Respiratory |  | Buoyancy Aid |  |  |
|  | Safety Footwear |  | Eye |  | Protective Clothing |  |  |

Section 5 – Permit to Work Required  Tick relevant box(es)

|  | Permit to Work Required | Permit Issued |
| --- | --- | --- |
|  | Confined Space |  |
|  | Hot/Cold Work |  |
|  | Other: |  |

Section 6 – Additional Comments

Section 7 – Assessed / Briefed

|  | Print Name | Signature |
| --- | --- | --- |
| Assessed by: |  |  |
| Briefed to: |  |  |

### 2.6 CG77 POLREP

This form should be completed and sent to the MCA within 12 hours of verbal communication, using the email address specified in Section 27: Zone26@hmcg.gov.uk.

### 2.7 JESIP M/THANE Form

## 3.0 Action Response Flow Charts

### 3.1 Fire On Quayside – General.

### 3.2 Fire – Ship Whilst Alongside Berth

### 3.3 Fire / Grounding / Sinking Or Capsizing Of A Vessel in Harbour Limits.

### 3.4 Actions Assuming Vessel Not Berthing

### 3.5 Actions If Vessel Is Able To Come Alongside A Berth

### 3.6 Fire Or Explosion On A Vessel Carrying Explosive/Hazardous Materials

#### 3.6.1 Actions If Vessel Is Alongside A Berth

#### 3.6.2	Actions If Vessel Underway or at anchor in Harbour Limits

### 3.7 Incident Involving Damage to Harbour Infrastructure

# Appendices

## 1.0 Dangerous Goods / Explosive Placards

### 1.1 I.M.D.G. Codes & Interpretation.

- A chart showing key IMDG codes.

# 2.0 Dangerous Goods – Consequences of Incidents

The following are consequences:

• Flammable gas cloud.

• Explosive reaction inside and outside buildings.

• Thermal radiation (flash fire or fireball).

• Blast waves.

• Flying debris.

• Pool fire if leakage from pipeline or tank.

• Jet fire if leakage from pipeline or tank.

• Flash fire.

Comprises: Gases which at 20ÅãC and a standard pressure of 101.3 kPa are ignitable when in a mixture of 13% or less by volume with air; or have a flammable range with air of at least 12 percentage points regardless of the lower flammable limit.

## 2.1 Non-Toxic & Non-Flammable Gases.

The following are consequences:

• Asphyxiating atmosphere in confined spaces.

• Toxic gas cloud.

Comprises: Gases which: 1) are asphyxiant – gases which dilute or replace the oxygen normally in the atmosphere; or .2) are oxidizing – gases which may, generally by providing oxygen, cause or contribute to the combustion, of other material more than air does; or .3) do not come under the other classes.

## 2.2 Poison Gases.

The following are consequences:

• Poisonous gas clouds.

• Fire.

• Thermal radiation.

• Flammable.

• Biological infection.

Comprises: Gases which are known to be so toxic or corrosive to humans as to pose a hazard to health; or are presumed to be toxic or corrosive to humans because they have a LC50 value equal to or less than 5,000 mℓ/m3 (ppm).

## 2.3 Flammable Liquids.

The following are consequences:

• Leakage from tank or pipeline, which results in flammable gas cloud.

• Pool fire.

• Flash fire.

• Explosion inside and outside buildings.

• Toxic gas cloud.

• Thermal radiation.

• Blast waves.

• Flying debris.

Comprises: Liquids, or mixtures of liquids, or liquids containing solids in solution or suspension (such as paints, varnishes, lacquers, etc., but not including substances which, on account of their other dangerous characteristics, have been included in other classes), which give off a flammable vapour at or below 60 C closed-cup test (corresponding to 65 C open-cup test), normally referred to as the “flashpoint”.

This also includes: 1) liquids offered for transport at temperatures at or above their flashpoint; and 2) substances transported or offered for transport at elevated temperatures in a liquid state, which give off a flammable vapour at temperatures equal to or below the maximum transport temperature. This class also comprises liquid desensitized explosives which are explosive substances that are dissolved or suspended in water or other liquid substances, to form a homogeneous liquid mixture to suppress their explosive properties.

## 2.4 Flammable Solids.

The following are consequences:

• Fire / spontaneous combustion.

• Explosion.

• Blast waves.

• Flying debris.

• Thermal radiation.

• Substances which, in contact with water, emit flammable gases.

• Thermal radiation.

• Toxic gas clouds.

Comprise: Readily combustible solids, (fibres, powdered, granular, or pasty substances) which can be easily ignited by brief contact with an ignition source such as a burning match. The danger may come not only from the fire but also from toxic combustion products. Metal powders are especially dangerous because of the difficulty of extinguishing a fire, since normal extinguishing agents such as carbon dioxide or water can increase the hazard.

## 2.5 Spontaneous Combustible.

The following are consequences:

• Fire.

• Explosion.

• Toxic gas cloud.

Comprise: Thermally unstable substances liable to undergo a strongly exothermic decomposition even without participation of oxygen (air). Self-reactive substances are classified into seven types according to the degree of danger they present. The decomposition of self-reactive substances can be initiated by heat, contact with catalytic impurities (such as acids, heavy-metal compounds, bases), friction or impact. The rate of decomposition increases with temperature and varies with the substance. Decomposition, particularly if no ignition occurs, may result in the evolution of toxic gases or vapours. For certain self-reactive substances, the temperature shall be controlled. Some self-reactive substances may decompose explosively, particularly if confined. This characteristic may be modified by the addition of diluents or by the use of appropriate packaging. Some self-reactive substances burn vigorously.

### 2.5.1 Substances Liable to Spontaneous Combustion.

The following are consequences:

• Fire.

• Endothermic reaction.

Comprise: 1) Pyrophoric substances, which are substances, including mixtures and solutions (liquid or solid), which, even in small quantities, ignite within 5 minutes of coming into contact with air. These substances are the most liable to spontaneous combustion;

2) Self-heating substances, which are substances, other than pyrophoric substances, which, in contact with air without energy supply, are liable to self-heating. These substances will ignite only when in large amounts (kilograms) and after long periods of time (hours or days).

## 2.6 Dangerous When Wet.

The following are consequences:

• Fire.

• Explosion.

• Blast waves.

Comprise: Either liquids or solids which, by interaction with water, are liable to become spontaneously flammable or to give off flammable gases in dangerous quantities. Certain substances, in contact with water, may emit flammable gases that can form explosive mixtures with air. Such mixtures are easily ignited by all ordinary sources of ignition, for example naked lights, sparking hand tools or unprotected light bulbs. The resulting blast wave and flames may endanger people and the environment.

## 2.7 Oxidizers.

The following are consequences:

• Thermal radiation.

• Blast waves.

• Flying debris.

• Fire.

• Explosion.

Comprise: Substances which, while in themselves are not necessarily combustible, may, generally by yielding oxygen, cause, or contribute to, the combustion of other material.

• Substances which in certain circumstances directly or indirectly evolve oxygen. For this reason, oxidizing substances increase the risk and intensity of fire in combustible material with which they come into contact.

• Mixtures of oxidizing substances with combustible material and even with material such as sugar, flour, edible oils, mineral oils, etc., are dangerous. These mixtures are readily ignited, in some cases by friction or impact. They may burn violently and may lead to explosion.

• There will be a violent reaction between most oxidizing substances and liquid acids, producing toxic gases.

• Toxic gases may also be evolved when certain oxidizing substances are involved in a fire.

## 2.8 Organic Peroxides.

The following are consequences:

• Be liable to explosive.

• Burn rapidly.

• Be sensitive to impact or friction.

• React dangerously with other substances.

• Cause damage to the eyes.

Comprise: Organic substances which contain the bivalent –O–O– structure and may be considered derivatives of hydrogen peroxide, where one or both hydrogen atoms have been replaced by organic radicals. Organic peroxides are thermally unstable substances which may undergo exothermic self-accelerating decomposition.

## 2.9 Poisons.

The following are consequences:

• Harmful to health.

• Dangerous when in contact with skin.

• Dangerous when inhaled.

• Toxic gases.

Comprise: Substances liable either to cause death or serious injury or to harm human health if swallowed or inhaled, or by skin contact.

• The dangers of poisoning which are inherent in these substances depend upon contact with the human body, that is by inhalation of vapours by unsuspecting persons at some distance from the cargo or the immediate dangers of physical contact with the substance.

• Nearly all toxic substances evolve toxic gases when involved in a fire or when heated to decomposition.

## 2.10 Infectious Substances.

The following are consequences:

• Infectious.

• Causes diseases.

• Causes permanent disability.

• Toxic gases.

Comprise: Substances which are known or are reasonably expected to contain pathogens. Pathogens are defined as micro-organisms (including bacteria, viruses, rickettsiae, parasites, fungi) and other agents such as prions, which can cause disease in humans or animals. An infectious substance which is transported in a form that, when exposure to it occurs, is capable of causing permanent disability, life-threatening or fatal disease in otherwise healthy humans or animals.

## 2.11 Radioactive.

The following are consequences:

• Radioactive emission.

Comprises: Any material containing radionuclides, where both the activity concentration and the total activity in the consignment exceed specified values. Radioactive material shall be assigned to one of the specified U.N. numbers depending on the activity level of the radionuclides contained in a package, the fissile or non-fissile properties of these radionuclides, the type of package to be presented for transport, and the nature or form of the contents of the package, or special arrangements governing the transport operation.

## 2.12 Corrosive.

The following are consequences:

• Skin and eye burns.

• Thermal radiation.

• Toxic gas clouds.

• Poisoning.

• Destructive effect on metals and fabrics.

• Corrosive to glass and earthenware.

Comprise: Substances which, by chemical action, will cause severe damage when in contact with living tissue or, in the case of leakage, will materially damage, or even destroy, other goods or the means of transport. Some of them can cause (severe) burns to skin, eyes and mucous membranes. Many substances are sufficiently volatile to evolve vapour irritating to the nose and eyes. A few substances may produce toxic gases when decomposed by very high temperatures. When involved in a fire, they evolve toxic gases. Poisoning may result if they are swallowed, or if their vapour is inhaled; some of them even may penetrate the skin. All substances in this class have a more or less destructive effect on materials such as metals and textiles. A few substances in this class can corrode glass, earthenware and other siliceous materials. Many substances in this class only become corrosive after having reacted with water, or with moisture in the air. The reaction of water with many substances is accompanied by the liberation of irritating and corrosive gases. Such gases usually become visible as fumes in the air. A few substances in this class generate heat in reaction with water or organic materials, including wood, paper, fibres, some cushioning materials and certain fats and oils.

## 2.13 Miscellaneous Dangerous Substances.

The following are consequences:

• Fire

• Leakage

• Thermal radiation.

• Carcinogenic particles.

Comprise: Substances which, on inhalation as fine dust, may endanger health (asbestos, etc.). Substances evolving flammable vapour (polymeric beads, plastic moulding compounds, etc.) together with lithium batteries, electric double layer capacitors, lifesaving appliances (air bags inflators, seat belt pretensioners), substances and articles which, in the event of fire, may form dioxins, substances transported or offered for transport at elevated temperatures, environmentally hazardous substances and genetically modified microorganisms (GMMO s) and genetically modified organisms.

The basic elements for classification of environmentally hazardous substances (aquatic environment) are: aquatic toxicity, potential for or actual bioaccumulation; and degradation (biotic or abiotic) for organic chemicals. Marine pollutants are transported under appropriate coding, according to their properties.

# 3.0 References

The Dangerous Goods in Harbours Regulations 2016. https://www.legislation.gov.uk/uksi/2016/721/made

Devon County Council Emergency Response Plan. https://www.devon.gov.uk/emergencies/emergency-response/

Devon County Council Rabies Contingency Plan.https://www.dcisprepared.org.uk/media/2139/animal-health-plan-lrfdcios-20131118-v12.pdf

Trading Standards Contingency plan for Landing of Livestock. https://www.devonsomersettradingstandards.gov.uk/tsi-single/?frmClient=C714B873-1185-6B25-FCF0CA3BD06B045D&frmItemID=300991&frmShared=1

Ilfracombe Harbour Oil Spill Contingency Plan 2022. This is a restricted document

Fire and Rescue Service Tactical Information Plan. This is a restricted document

Local Resilience Forum Combined Agency Emergency Response Plan. https://northdevon.gov.uk/community-safety-and-emergencies/emergency-planning/

Dangerous Vessel Act 1985.	https://www.legislation.gov.uk/ukpga/1985/22?view=extent

Devon County Council Emergency Telephone Directory. This is a restricted document